MidiVerb 4

System Exclusive Format

Software Version 1.03

March 17, 1995

## OVERVIEW

All MidiVerb 4 system exclusive messages have the following format:

$F0		System Exclusive status

$00 $00 $0E	Alesis manufacturer ID

$11		MidiVerb 4 ID

nn		Channel

cc		Opcode

dd		Data

:		  :

:		  :

$F7		End of Exclusive

## THE CHANNEL BYTE

For messages originating from the MidiVerb 4 the system exclusive channel will always be the same as the standard MIDI channel, and can be adjusted by the user via the global utility functions.  If the user has enabled Omni mode the MidiVerb 4 will respond to system exclusive messages on ANY channel and transmit on channel 1, just as it does for ordinary channel messages (note, controller, program change, etc.) when Omni mode is enabled.

If a transmitting device such as a computer does not know the MIDI channel of a MidiVerb 4 it can use transmit the MIDI universal system exclusive inquiry message with the channel byte set to $7F (ALL CALL).  The MidiVerb 4 will respond with a universal system exclusive identity message with the channel byte set to the current MIDI channel (0-15).  This is the recommended technique for closed loop systems as it allows software programs to confirm the MidiVerb 4's software version and revision while it finds the channel.

## THE OPCODE BYTE

Opcodes are divided into two categories:  Requests and Dumps.  The values for Request opcodes are closely related to their corresponding Dump opcodes -- they are simply the Dump opcode with bit 6 set.

Only one of the Request opcodes has a data byte -- the Single Program request.  The data byte specifies which User Program (0-127) it is requesting.

Dump		Request	Function				Dump Message Size

Opcode	Opcode	Description				(in bytes)

$00		$40		System Info				22

$01		$41		Program Table			264

$02		$42		Edit Buffer				50

$03		$43		All User Programs			5384

$04		$44		Single Program			51

$05		None		Store Program				9

## SYSTEM INFO DATA

The System Info data is as follows:

Input Volume Stereo			0-100

Input Volume Ch 1			0-100

Input Volume Ch 2			0-100

Output Volume Stereo		0-100

Output Volume Ch 1			0-100

Output Volume Ch 2			0-100

Mod X					see Modulator Definitions

Mod X Amplitude High Nibble	see Modulator Amplitude

Mod X Amplitude Low Nibble		:

Mod Y					see Modulator Definitions

Mod Y Amplitude High Nibble	see Modulator Amplitude

Mod Y Amplitude Low Nibble		:

Footswitch Mode			see Footswitch Modes

System Status Byte			see System Status Byte

Modulator Definitions

0	Pitch Bend

1	Aftertouch

2	Note Number

3	Velocity

4	Controller 0

:		:

123	Controller 119

Modulator Amplitude

The modulation amplitude is nibblized with the high nibble being sent first.  The value is in offset binary, so the values 0 to 98 represent amplitudes -99 to -1, the value 99 represents amplitude 00, and the values 100 to 198 represent amplitudes 1 to 99.

Footswitch Modes

0	Advance

1	Bypass

2	Controller

System Status Byte

Bit 0	Thru (0 = Off, 1 = On)

Bit 1	Omni

Bit 2	Bypassed

Bit 3	Cascade

Bit 4	Comparing

Bit 5	DON'T ALTER

Bit 6	Edited

Bit 7	0

## PROGRAM TABLE DATA

The Program Change Table data consists of 256 transmitted bytes.  This results in 128 actual bytes, since the data is nibblized and transmitted as follows:

0000dddd	first byte contains high nibble (bits 4-7)

0000dddd	second byte contains low nibble (bits 0-3)

Values 0-127 correspond to Preset Programs 00-127; values 128-255 correspond to User Programs 00-127.

## EDIT BUFFER DATA

The Edit Buffer data consists of one Program Data Block, defined below.

Program Data Block

Reserved		Always zero

Configuration		0-31

8-Byte Name

16 Parameters		nibblized so transmit as 32 bytes

## ALL USER PROGRAMS DATA

The All User Programs data consists of 128 Program Data Blocks, defined above.

## SINGLE PROGRAM DATA

The Single Program data is defined as follows:

Program number	0-127 (selects User 00-127)

Program Data Block

## STORE PROGRAM DATA

This message has no corresponding request and contains a single byte of data.  It instructs the MidiVerb 4 to store the current contents of the Edit Buffer into a specified User memory location.  The data byte contains the User Program number 0-127.

## NOTES

When you send MIDI Sysex "All" from the MidiVerb 4's Util Mode this sends four messages total:  System Info, Program Table, Edit Buffer, and All User Programs.